# Node Drain RCA

GPU node 98dci4-gpu-0003 — five drain events, 26 May – 1 June 2026

10 June 2026

# Verdict

The drains were caused by an environmental, node-level condition — a kernel-driver wait that occasionally fails to resolve — not by any user code, and not by scancel, SIGKILL, or SLURM --time expiry. The fix is admin-side configuration plus lightweight forensic instrumentation.

# Mechanism (proven)

- A process enters uninterruptible D-state: a kernel sleep inside the NVIDIA driver (os_acquire_rwlock_*, uvm_*), the CXI Slingshot fabric (cxiWaitEventWait), or the GPFS client.

- The D-state outlives KillWait (30 s) + UnkillableStepTimeout (60 s) — values confirmed from scontrol show config.

- slurmstepd reports “Kill task failed”; slurmctld sets the node to DRAINING. Recovery requires a manual nvidia-smi --gpu-reset or reboot, then scontrol update … state=resume.

This is documented SLURM behaviour working as designed. The open question was only ever what makes a D-state last more than 90 seconds instead of the normal sub-millisecond.

# Root cause

A stochastic driver/firmware disposition of this node toward occasionally sustained kernel waits, outside user-space control:

- Independent of code. Five drains in 8 days were triggered by at least two unrelated user groups running different software — including one drain that predates our GPU work. Same node, same failure signature.

- Not reproducible. Sixteen deliberate attempts to drain the node — infinite GPU kernels, driver-lock deadlock constructions, SIGKILL mid-collective, --time expiry into a live hung NCCL process, and the verbatim code of a job that had previously drained the node — produced zero drains.

- D-state is normal and transient. An independent observer captured 10,514 process-state samples during live GPU work: hundreds of D-state entries across all three driver surfaces, all sub-second, all reaped cleanly.

# Ruled out by measurement

| Suspected trigger | Result under test |
| --- | --- |
| scancel / SIGKILL of a CUDA/NCCL job | Clean kill every time (collectives are killable R-state) |
| SLURM --time expiry | Clean TIMEOUT, even into a hung NCCL process |
| NCCL teardown (ncclCommAbort) | Interruptible S-state; signals land in < 1 s |
| Blocked network I/O on the firewalled node | Interruptible S-state |
| Cascade from a GPU pre-wedged by an earlier failure | Refuted — job 1209813 drained a clean node |

The defensive rules built on these suspicions (never-scancel, STOP-FILE cancellation contract, three-layer time-limit pattern) were removed from the project on 10 June 2026.

# Recommendations

Admin-side (the actual fix):

- Epilog GPU reset on job completion, clearing residual contexts before the next allocation.

- Auto-resume policy (HealthCheckProgram or cron): resume the node automatically when GPUs check out clean — turns a multi-hour outage into minutes.

- Review UnkillableStepTimeout (60 s → ~120 s) so transient D-state spikes cannot cross the drain threshold.

User-side: instrument rather than restrict. Every long GPU job now runs a SIGTERM-immune sidecar that samples cgroup process states (state + kernel wait symbol), GPU status, and kernel Xid events once per second to shared storage. If a drain recurs, the offending PID and kernel symbol are captured automatically and can be attached to the admin ticket.